# Review

Project Title: Multi-Fidelity 3D Aero Surrogate

## 1. Executive Summary

This report defines the architecture for an aerodynamic surrogate platform designed to bridge the gap between Blended Wing Bodies (BWB) and Conventional Fixed-Wing UAVs. The system operates in two modes: Inverse Design (finding the best wing for a mission) and Forward Analysis (predicting physics for a user-provided mesh). The core strategy relies on a Hybrid Input Vector (Planform + CST) and a Multi-Fidelity Training pipeline to ensure high accuracy with minimal CFD cost.

## 2. Geometric Representation: The Hybrid Input Stack

To represent the transition from a wide, integrated body (BWB) to a slender wing, we use a two-part input vector. This separates "global layout" from "local curvature."

### Component 1: Planform Parameters (The "Skeleton")

- Variables: Wingspan (b), Aspect Ratio (AR), Taper Ratio (λ), Sweep Angle (Λ), and Dihedral (Γ).

- Role: These define the macroscopic layout. By providing these as explicit inputs, the AI does not have to "deduce" the wing size from abstract coefficients, improving interpolation stability.

### Component 2: 3D Class Shape Transformation (The "DNA")

- Variables: A matrix of Bernstein polynomial coefficients (Bi,j​).

- Role: Defines the cross-sectional thickness, camber, and spanwise twist distribution.

- The "BWB" Advantage: CST is a continuous mathematical function. It allows the center of the aircraft to "bulge" into a fuselage or "thin" into a wing within a single unified coordinate system (u,v).

## 3. The Multi-Fidelity (MF) Learning Strategy

High-fidelity CFD is too expensive to sample the entire design space. We use a Hierarchical Delta-Learning approach.

### The Fidelity Split

- Low-Fidelity (VLM - 80% of data): 2,000+ samples. Provides the "trend." It teaches the AI the linear lift-slope and basic induced drag patterns.

- High-Fidelity (CFD - 20% of data): 100–300 samples. Provides the "correction." It teaches the AI about viscous drag, flow separation at the BWB-wing junction, and transonic shockwaves.

### The "Delta" Architecture

The model is trained to predict:

Result=ModelVLM​(X)+ΔError​(X)

Where ΔError​ is the specific difference between VLM and CFD. This ensures that even with few CFD runs, the model remains physically grounded by the VLM's global trends.

## 4. Operational Pipelines

### Pipeline A: Optimization (Requirements → Best Wing)

- User Input: Target CL​, CD​ limits, and internal volume requirements.

- Core Model: A fast Multi-Layer Perceptron (MLP).

- Process: An optimization algorithm (e.g., Genetic Algorithm or Bayesian Opt) iterates through the Input Vector. The MLP predicts the performance of 10,000 designs in seconds to find the global optimum.

### Pipeline B: Analysis (Uploaded Mesh → Results)

- User Input: Raw STL/OBJ mesh.

- The Fitting Step: A "Reverse Engineering" algorithm fits the uploaded mesh to a 3D-CST + Planform vector using Least-Squares optimization.

- Field Prediction: A Convolutional Neural Network (CNN) treats the wing as a "Shared Topology" (a 64×64 grid). It predicts the Pressure Field (Cp​) across the surface, which is then re-projected back onto the user's original STL.

## 5. Critical Technical Risks & Mitigation

| Risk | Description | Mitigation Strategy |
| --- | --- | --- |
| Representational Error | CST might "smooth out" local features (like sharp kinks) that affect drag. | Use Piecewise CST blocks for the center-body and outer-wing to capture localized curvature. |
| The Curse of Dimensionality | Having >50 inputs (CST + Planform) makes the design space too large to search. | Use Active Subspaces to identify which 10–12 variables actually drive 90% of the performance change. |
| Extrapolation Failure | The AI will "hallucinate" incorrect physics if asked to predict a wing far outside the training set. | Implement Active Learning. If the model's uncertainty is high, automatically trigger a new CFD run to "fill the gap." |

## 6. Implementation Checklist for the Team

- [ ] Geometry Script: Develop the 3D-CST generator that accepts Planform scalars.

- [ ] Data Gen: Set up an automated pipeline (OpenVSP for VLM, OpenFOAM/SU2 for CFD).

- [ ] Mapping: Create the "Shared Topology" mapper to turn 3D CFD results into 64×64 pressure images.

- [ ] Model Training: Build the Multi-Fidelity Delta-Network in PyTorch.

- [ ] The "Fitter": Develop the STL-to-CST optimization script for user uploads.

Conclusion: This architecture avoids the "Big Data" requirement of pure Graph Neural Networks while maintaining the physical rigor of CFD. By using CST as the mathematical bridge, the platform can seamlessly transition between BWB and Fixed-Wing designs, providing a truly unified design environment for the next generation of UAVs.

## Το Gemini είπε

# Considerations

Yes — for your design surrogate, using CST directly is sensible.
No — for a mesh GNN, replacing the encoder with CST is usually the wrong move.

That’s the clean answer.

## 1) Your instinct is not stupid at all

If the geometry is already represented by:

- planform scalars

- CST coefficients

- flow variables

then for a global surrogate that predicts things like:

- CL​

- CD​

- Cm​

- maybe volume, static margin, etc.

you can absolutely do:

vars[CST+flow vars]→MLP→[CL​,CD​,Cm​]

That is a normal and efficient architecture for conceptual design. In your case, it is probably the right default.

The trap begins only when you move to a mesh-based field predictor.

## 2) Why CST cannot simply replace the mesh encoder in a GNN

In a mesh GNN, the model predicts quantities at many points on the surface, not just one global scalar. NVIDIA’s MeshGraphNet-style workflows operate on graphs built from geometry/mesh connectivity, where node and edge attributes are encoded before message passing. Their external aerodynamics examples target outputs such as surface pressure and wall shear, not just global coefficients.

That means the model needs local state per node.

So each node needs something like:

- where am I?

- what is my normal?

- who are my neighbors?

- what is the flow condition?

CST is global.
A node embedding is local.

Those are not interchangeable.

If you inject the same CST vector into every node and say “done, no encoder,” you are basically forcing the model to reconstruct local geometry from a repeated global code. That is clumsy and usually wasteful.