### PHASE 0: Hardware & Operational Scope

To make this work with just 1 PC and 1 Laptop, we must strictly constrain the aerodynamic envelope. If the AI has to learn everything from slow drones to supersonic jets, it will fail.

- Hardware Allocation:

  - The PC (Data Factory): Runs OpenFOAM (CFD) and VSPAERO (VLM) 24/7. Trains the heavy Graph Neural Network (GNN).

  - The Laptop (The Brain): Used for coding in Cursor, running the optimization algorithms, and training the fast MLPs.

- The Prototype Flight Envelope:

  - Velocity: 15 m/s to 30 m/s (Mach ~0.04 to 0.08).

  - Angle of Attack ($\alpha$): 0° to 8° (keeps physics mostly attached; avoids chaotic deep stall).

### PHASE 1: The Hybrid Input Vector (17 Variables)

This is the mathematical foundation of your Optimization Engine. Every wing or BWB generated by your system is flattened into exactly 17 numbers.

| Category | Variable | Bounds / Prototype Range |
| --- | --- | --- |
| Planform | Wingspan ($b$) | 1.2 to 1.8 meters |
| Planform | Aspect Ratio ($AR$) | 5.0 to 7.0 |
| Planform | Taper Ratio ($\lambda$) | 0.3 to 0.6 |
| Planform | Sweep Angle ($\Lambda$) | 20° to 35° |
| Planform | Dihedral Angle ($\Gamma$) | 0° to 5° |
| CST (Upper) | Upper Weights ($Wu_0 ... Wu_4$) | 5 coefficients (4th-order Bernstein) |
| CST (Lower) | Lower Weights ($Wl_0 ... Wl_4$) | 5 coefficients (4th-order Bernstein) |
| Flight State | Mach Number ($M$) | 0.04 to 0.08 |
| Flight State | Angle of Attack ($\alpha$) | 0° to 8° |

### PHASE 2: Codebase Architecture & Modules

You will instruct your AI agent to build the software exactly according to this directory tree.

- geometry/

  - cst_generator.py: Converts the 17-variable array into a watertight 3D STL mesh using the geomdl library.

  - reverse_fitter.py: Uses scipy.optimize.minimize (Least-Squares) to force an uploaded STL into the 15 geometric parameters.

- simulation/

  - run_vspaero.py: Automates VLM runs.

  - run_openfoam.py: Automates blockMesh, snappyHexMesh, and simpleFoam (Spalart-Allmaras turbulence model).

- models/

  - delta_mlp.py: The fast Multi-Layer Perceptron (Track A).

  - mesh_gnn.py: The Graph Network Simulator (Track B).

- utils/

  - graph_builder.py: Converts STLs into PyTorch Geometric graph structures.

### PHASE 3: The Data Factory (Multi-Fidelity Generation)

You cannot train an AI without data. Your PC will spend weeks generating this dataset.

- Generate the Trend (Low-Fidelity):

  - Use Latin Hypercube Sampling to generate 2,000 to 5,000 distinct 17-variable vectors.

  - Run them through the VLM. Extract $C_L$, $C_D$, $C_{M,y}$.

- Generate the Truth (High-Fidelity):

  - Select 100 to 300 highly diverse designs from the LF set.

  - Run them through RANS CFD.

  - Extract global coefficients ($C_L, C_D, C_{M,y}$) AND the localized surface pressure field ($C_p$).

### PHASE 4: Track A – The Design Optimization Engine

This is your primary tool for "Inverse Design" (finding the best wing).

- The Delta Math: Calculate the error between CFD and VLM for your 300 high-fidelity runs: $\Delta_{Error} = CFD - VLM$.

- The MLP Architecture: Build a standard PyTorch MLP (e.g., 3 hidden layers of 128 neurons).

  - Input: The 17-variable vector.

  - Target Output: $\Delta C_L$, $\Delta C_D$, $\Delta C_{M,y}$.

- The Deployment: Hook this MLP into a Genetic Algorithm. When searching for a new UAV design, the system calculates $Result = VLM(X) + MLP(\Delta_{Error})$. It can evaluate 10,000 designs in seconds.

### PHASE 5: Track B – The Physics Engine (GNN Fallback)

This is the safety net for "Forward Analysis." If a user uploads a shape with sharp kinks or weird pods, the reverse_fitter.py will fail to match it to a CST (geometric error > 2%). The system routes the raw mesh here.

- Graph Construction: * Node Features: $[x, y, z, n_x, n_y, n_z, M, \alpha]$.

  - Edge Features: $[dx, dy, dz, \text{distance}, \text{face\_normal}]$.

- The GNN Architecture (Encoder-Processor-Decoder):

  - Encoder (MLP): Translates physical node/edge features into latent vectors (dimension 128).

  - Processor (Message Passing): Passes data between connected nodes for 3 to 5 layers. The nodes "learn" the local topology (e.g., "I am behind a curved leading edge").

  - Decoder (MLP): Translates the updated latent vector into a specific $C_p$ value for that node.

- The Output: A highly accurate 3D pressure map. Integrate the pressures over the surface mesh to calculate the final $C_L, C_D, C_{M,y}$.

### PHASE 6: Agentic Execution Strategy

You are the Lead Engineer. Claude (via Cursor/Windsurf) is your Junior Developer.

- Do not ask for the whole project at once. AI context windows will break.

- Verify Physics Manually: The AI will write perfect PyTorch code, but it will likely screw up the OpenFOAM fvSchemes or boundary conditions. You must manually run and check the first few CFD cases to ensure the fluid dynamics are sound.

- Step-by-Step Prompting: Build Phase 2 (cst_generator.py) first. Do not move to Phase 3 until you can successfully generate and view a watertight STL from a 15-number array.